Synopsis

DIGITAL TWIN-BASED MICROGRID MONITORING AND PROBABILISTIC BLACKOUT PREDICTION USING MACHINE LEARNING

By

Saidu Venkata Revanth Varma       [BT23CSA003]

Vankadoth Praveen                          [BT23CSA037]

Manikanta Bojja                              [BT23CSA043]

Semester: 6th		     Branch: CSE-AIML

Under the guidance of

Dr. Khushboo Jain

Department of Computer Science and Engineering

Indian Institute of Information Technology Nagpur - 441108 (India)

© Indian Institute of Information Technology (IIIT) 2026

- Introduction:

The increasing use of renewable energy sources and distributed generation has significantly transformed modern power distribution systems into complex microgrids. While microgrids offer improved reliability and sustainability, their operation is complicated by the intermittent nature of renewable energy sources, dynamic demand fluctuations, and voltage stability issues.

A digital twin integrates system models, cognitive analytics, and historical data to create a virtual representation of a physical system. Digital twins enable offline simulation, forecasting, and scenario analysis in power systems without requiring the real-time hardware or field deployment. When integrated with machine learning approaches, digital twins can aid predictive monitoring and blackout risk assessment under varying operating conditions.

This project depicts an offline digital twin-based framework for microgrid monitoring and blackout prediction. The IEEE 33-bus distribution system serves as the physical network model, while real-world historical load and solar generation datasets are employed to capture realistic operating trends. Machine learning models are being developed for load forecasting, solar power forecasting, and blackout risk prediction. The proposed system operated entirely within an offline simulation environment, which makes it ideal for academic study and reliability analysis.

- Goals:

The main goals of this project are: The main goals of this project are:

- To make an offline digital twin of a microgrid using the IEEE 33-bus distribution network. To make an offline digital twin of a microgrid using the IEEE 33-bus distribution network.

- To combine data-driven load and solar power patterns with the physical limits of the network. To combine data-driven load and solar power patterns with the physical limits of the network.

- To use AC power flows to model energy balance, voltage behavior, and blackout situations. To use AC power flows to model energy balance, voltage behavior, and blackout situations.

- To create blackout labels based on physics using deterministic power system constraints. To create blackout labels based on physics using deterministic power system constraints.

- To train a machine learning model that predicts the chance of a blackout and captures nonlinear risk patterns. To train a machine learning model that predicts the chance of a blackout and captures nonlinear risk patterns.

- To examine microgrid reliability across various operational scenarios. To examine microgrid reliability across various operational scenarios.

- Dataset Overview:

The project utilizes 3 datasets for unique use cases in the Digital Twin structure:

  - Load Dataset:

- Source: UCI Machine Learning Repository

- Dataset: Electricity Load Diagrams 2011–2014

- Description: This includes historical electricity consumption data from several consumers with 15-minute resolution.

- Use: Normalized by IEEE 33-sectioned system base loads for load forecasting and simulation and summarized to produce real-life profiles.

  - Solar Power Dataset:

- Source: Kaggle

- Data Set: Data on Solar Power Generation

- Description: This document has information about past solar power generation and environmental factors like temperature and irradiation.

- Use: Used to figure out solar generation patterns, which were then adjusted and scaled to a realistic level of photovoltaic penetration for solar forecasting and microgrid simulation.

  - Microgrid Network Model:

- IEEE 33-Bus Distribution System Model

- Tool: pandapower

- Description: A standard benchmark radial distribution network for looking at voltage and power flow.

- Use: It is the physical layer of the digital twin that is used to simulate power flow and check voltage limits.

4. Methodology

The proposed system implements a Digital Twin-based framework for monitoring and predicting blackout risk in microgrids. The methodology integrates data-driven temporal patterns, power system simulation, and machine learning-based risk prediction.

The system operates completely in an offline simulation environment.

4.1 Data Acquisition

The project utilizes three major data sources.

Load Dataset

Source: UCI Machine Learning Repository

Dataset: Electricity Load Diagrams 2011–2014

Description:

This dataset contains electricity consumption data collected from 370 individual consumers at 15-minute intervals.

Each record represents the power consumption (kWh) of a consumer at a specific time.

Use in the project:

The dataset is used to extract realistic electricity demand patterns that represent daily and seasonal variations in load demand.

These patterns are later scaled to match the base load of the IEEE 33-bus distribution system.

Solar Power Dataset

Source: Kaggle

Dataset: Solar Power Generation Data

Description:

This dataset contains solar plant generation data along with environmental parameters such as:

- AC Power

- DC Power

- Irradiation

- Module temperature

- Ambient temperature

Use in the project:

The dataset is used to extract realistic solar generation patterns that represent photovoltaic generation variability.

The power output from multiple inverters is aggregated and converted into hourly solar generation profiles.

Microgrid Network Model

Network Model: IEEE 33-Bus Distribution System

Simulation Tool: pandapower

Description:

The IEEE 33-bus system is a standard radial distribution network commonly used for voltage stability and power flow analysis.

Use in the project:

The IEEE network serves as the physical layer of the digital twin, where load demand and solar generation are injected and AC power flow simulations are performed.

4.2 Data Preprocessing

The datasets undergo preprocessing before they are used in the digital twin simulation.

Load Data Processing

Steps performed:

- Parse timestamps from the dataset.

- Aggregate consumption data across consumers to obtain overall demand patterns.

- Convert 15-minute resolution data to hourly averages.

- Handle missing values and remove inconsistencies.

- Normalize the load values between 0 and 1.

The normalized demand pattern is defined as:

LoadPattern(t)

This pattern captures real-world demand behavior including peak and off-peak periods.

Solar Data Processing

Solar dataset preprocessing includes:

- Parsing timestamps from generation data.

- Aggregating AC power output from all solar inverters.

- Converting 15-minute data into hourly generation profiles.

- Converting power values into MW units.

- Normalizing solar generation values between 0 and 1.

The resulting solar pattern is defined as:

SolarPattern(t)

This pattern reflects typical photovoltaic generation characteristics.

4.3 Digital Twin Simulation

The digital twin is implemented as an offline time-step simulation system.

Each simulation step represents one hour of microgrid operation.

For each time step t, the following steps are executed.

Load Injection

The base load of each bus in the IEEE 33-bus network is scaled using the load pattern.

Loadi(t)=BaseLoadi×LoadPattern(t)

Where:

- Loadi(t)is the load at bus i

- BaseLoadiis the original load value of the IEEE network.

Solar Power Injection

Solar power generation is injected into selected buses using the solar generation pattern.

PV(t)=PVcapacity×SolarPattern(t)

This models photovoltaic generation within the microgrid.

AC Power Flow Simulation

At each time step, AC load flow is solved using pandapower.

The simulation computes:

- Bus voltage magnitudes

- Line loading

- Power losses

- Power flow distribution

These outputs represent the operational state of the microgrid.

4.4 Power Balance Equation

The power balance in the microgrid is expressed as:

Pgeneration=Pload+Ploss

Where:

- Pgenerationrepresents total power generated

- Ploadrepresents total system demand

- Plossrepresents transmission losses.

Maintaining power balance is necessary for stable microgrid operation.

4.5 Voltage Constraint

Voltage limits are monitored to ensure network stability.

0.95≤V≤1.05

Where:

- Vrepresents bus voltage magnitude.

If bus voltages fall outside this range, the system is considered to be under voltage stress conditions.

4.6 Blackout Label Generation

Blackout labels are generated using deterministic power system constraints.

A system state is labeled as a blackout condition if:

- Bus voltage drops below the minimum limit

- Line loading exceeds allowable limits

- Large load-generation imbalance occurs.

These labels are used to train the machine learning model.

4.7 Feature Extraction

During simulation, system state variables are recorded as features.

Key features include:

- Hour of the day

- Load pattern value

- Solar generation value

- Total system load

- PV generation

- Net power imbalance

- Power losses

- Minimum bus voltage

- Maximum line loading.

These features represent the operational condition of the microgrid.

4.8 Machine Learning Model

The extracted features and blackout labels are used to train machine learning models such as:

- Random Forest

- XGBoost

- Logistic Regression.

The ML model learns the relationship between system operating conditions and blackout risk.

The model outputs a probability of blackout occurrence.

4.9 Reliability Analysis

The trained model is used to analyze microgrid reliability under different operating scenarios.

These scenarios include:

- High load demand

- Low solar generation

- Increased photovoltaic penetration

- Battery storage integration.

The results help identify high-risk operational periods and system vulnerabilities.

5. Expected Outcomes:

- A digital twin of a microgrid that works completely offline.

- Realistic behavior of voltage and power balance.

- Prediction of the risk of a blackout based on probability.

- Finding times when operations are at high risk.

- Reliability insights based on scenarios.

6. Tools and Technologies:

- Language for programming: Python

- Simulation of Power Systems: pandapower

- Libraries for machine learning: scikit-learn and XGBoost

- Data Processing: pandas, NumPy

- Matplotlib and Seaborn for visualization

- Jupyter Notebook and VS Code are the development environments.

There are no real-time hardware or hardware-in-the-loop (HIL) systems used in any of the simulations or ML models.

7. Conclusion:

This project shows how to use a physics-informed, data-driven digital twin to check for blackout risks and keep an eye on microgrids. The system captures nonlinear interactions between load demand, renewable generation, and network constraints by combining AC power flow simulation with machine learning. The proposed PHYSIC-DT-RISK framework offers probabilistic insights into blackout risk while maintaining complete offline functionality, reproducibility, and academic rigor. This method is good for educational research, and future extensions that involve real-time systems and advanced control strategies can build on it.